##### KTM

#### FUEL TANK:

2.7 GALLONS

IMS Tank ID #: 
KTM-12

Fits KTM Models

KTM: 250 SX-F (2006’)

KTM: 250 XC-F(2007’)

IMS Part #: 113322/213322

# IMS PARTS LIST

(1) Fuel Tank

(1) *Gas Cap & Seal

(1) Seat Hook Catch Fitting

7.9mm (Short)

(1) 6 x 16mm Button Head Bolt

(2) 6x12 With 8mm Head

10mm Flange Bolts (Petcock)

(8) 6x12mm With 8mm Head 13mm Flange Bolts (Shrouds)

(1) 6” Fuel Line

*(Excluded on Dry-Break )

OEM ITEMS TO USE

• Petcock

• Shrouds

WARNING!

Prior To Installation Clean Interior of IMS Tank Thoroughly.  Install

This Fuel Tank ONLY In a Well Ventilated Area, As Gasoline Fumes Are EXTREMELY Dangerous.

DO NOT START OR OPERATE THE VEHICLE IF THERE ARE ANY FUEL LEAKS

IMS Recommends Installation Be Performed By a Licensed Mechanic.

Improper Installation May Result In Bodily Injury Or Death.

Prior To Installation Read All Instructions Thoroughly.

### TANK REMOVAL

Drain all fuel from OEM fuel tank into a government approved fuel container with a greater capacity than fuel tank on vehicle.

Remove seat, fuel line at petcock, and shroud to radiator attachment bolts, remove shrouds, and tank bolt (center mount).

Remove petcock from OEM tank.

### TANK INSTALLATION

Mount seat hook fitting on topside of tank.  Install OEM petcock using the IMS 6x12 flange bolts.

Fit tank on the vehicle, install center-mounting bolt. Attach shrouds to the radiators then attach to the IMS tank. Attach the seat and fuel line.

Be sure that the tank is mounted securely and does not bind or in any way inhibit the controls or function of the vehicle.  Ensure that there are no sharp objects that rub against the tank that may eventually puncture the tank. Check that you have sufficient clearance from any heat source that could melt the fuel tank.

Turn the petcock to the closed position.  Fill you new IMS fuel tank, check for any leaks & enjoy your longer ride!

“FOR COMPETITION USE ONLY”

“Legal in California ONLY for racing vehicles

which may never be used upon a highway.”

| DISCLAIMER: | Due to uncontrolled variables in the manufacturing process of rotationally molded fuel tanks, the fuel tank may vary in size and shape by up to 7% of the manufacturer’s original listed capacity |
| --- | --- |